Experiment 2.2

Verification of Even and Odd Signal Decomposition

Aim

Decompose a signal into even and odd parts and verify reconstruction.

Apparatus Required

PC with MATLAB.

Program (MATLAB)

% Verification of Even and Odd Signal Decomposition

clc;

clear;

close all;

% Time axis

t = -5:0.1:5;

% Original Signal

x = t.^3 + t;

% Time-reversed Signal

x_rev = (-t).^3 + (-t);

% Even Component

xe = (x + x_rev)/2;

% Odd Component

xo = (x - x_rev)/2;

% Reconstruction of Original Signal

x_reconstructed = xe + xo;

% Plotting

figure;

subplot(4,1,1);

plot(t,x,'LineWidth',2);

grid on;

title('Original Signal x(t)');

xlabel('Time');

ylabel('Amplitude');

subplot(4,1,2);

plot(t,xe,'LineWidth',2);

grid on;

title('Even Component x_e(t)');

xlabel('Time');

ylabel('Amplitude');

subplot(4,1,3);

plot(t,xo,'LineWidth',2);

grid on;

title('Odd Component x_o(t)');

xlabel('Time');

ylabel('Amplitude');

subplot(4,1,4);

plot(t,x_reconstructed,'LineWidth',2);

grid on;

title('Reconstructed Signal x_e(t) + x_o(t)');

xlabel('Time');

ylabel('Amplitude');

Output Generated Graph

Original signal plus even (dashed) and odd (dotted) components.

Result

Signal equals sum of even and odd parts.