- SIGNAL GENERATION AND BASIC SIGNAL OPERATIONS

Experiment 1.3

Generation of Triangular and Sawtooth Signals

Aim

Generate and plot triangular and sawtooth signals.

Apparatus Required

PC with MATLAB.

Program (MATLAB)

% Generation of Triangular and Sawtooth Signals

clc;

clear;

close all;

% Time axis

t = 0:0.01:2;

% Sawtooth Signal

x1 = sawtooth(2*pi*t);

% Triangular Signal

x2 = sawtooth(2*pi*t,0.5);

% Plotting

figure;

subplot(2,1,1);

plot(t,x1,'LineWidth',2);

grid on;

title('Sawtooth Signal');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(2,1,2);

plot(t,x2,'LineWidth',2);

grid on;

title('Triangular Signal');

xlabel('Time (s)');

ylabel('Amplitude');

Output Generated Graph

Result

Triangular wave has equal rise and fall; sawtooth shows sharp drop and gradual rise.